Aerial Lift Inspection Checklist

Pre-Use / Periodic Inspection per OSHA 1926.453 and ANSI A92

Date: _______________

Inspector/Operator: _______________

Make/Model: _______________

Unit ID / Serial #: _______________

Lift Type: Boom / Scissor / Vertical Mast / Other

Inspection Type: Pre-Use / Monthly / Annual

Part 1: Base & Chassis

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Tires/wheels or tracks | Proper inflation (pneumatic). No cuts, bulges, excessive wear. Lug nuts tight. Foam-filled tires intact. Track tension correct. |  |  |  |
| Outriggers/stabilizers | Extend and retract fully. Pads present. Locks engage. Cylinders no leaks. Indicators functional. |  |  |  |
| Frame and turntable | No cracks, excessive corrosion, or structural damage. Turntable bearing no excessive play. Swing gear lubricated. |  |  |  |
| Drive system | Drive motors respond correctly. Brakes hold on slope. Speed selector functional. Steering responsive. |  |  |  |
| Battery/fuel system | Battery charged, connections tight, no corrosion. Fuel level adequate. No leaks. Emergency disconnect accessible. |  |  |  |

Part 2: Boom, Platform & Controls

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Boom sections | No cracks, dents, or deformation. Welds intact. Boom pins secured with keepers. Wear pads in spec. |  |  |  |
| Hydraulic system | No leaks at cylinders, hoses, fittings. Fluid level correct. Hoses no abrasion, cuts, or bulges. |  |  |  |
| Platform/bucket | Floor solid, no holes. Railings secure (42" top, 21" mid). Toe boards in place. Gate/chain closes. |  |  |  |
| Upper controls | All functions operate smoothly from platform. Emergency stop works. Speed proportional to input. Labels legible. |  |  |  |
| Lower controls | Override functions operational. Emergency lowering works. Key switch functional. Controls clearly labeled. |  |  |  |

Part 3: Safety Systems

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Load capacity | Capacity plate legible and visible. Platform load within rating. No unauthorized modifications. |  |  |  |
| Fall protection | Harness anchor point present and rated. Lanyard attachment tested. Harness required signage posted. |  |  |  |
| Limit switches | Height/reach limits function. Tilt alarm/cutout works. Overload protection engages at rated capacity. |  |  |  |
| Electrical safety | Insulation rated for voltage (if category A/B/C). No damage to insulating components. Proximity alarm works. |  |  |  |
| Placards and manuals | All warning decals present/legible. Operator manual in storage compartment. Capacity chart accessible. |  |  |  |

NEVER override safety devices. Always wear fall protection (harness + lanyard) attached to the manufacturer's anchor point. Do not belt off to adjacent structures unless specifically authorized. Electrocution is the leading cause of aerial lift fatalities - maintain safe distances from power lines.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Inspector/Operator: ____________________________    Date: ______________    Signature: ____________________________

Equipment Manager / Supervisor: ____________________________    Date: ______________    Signature: ____________________________