COMPRESSED GAS CYLINDER INSPECTION CHECKLIST

Per OSHA 29 CFR 1910.101 and CGA Pamphlet P-1

| Date: _______________ | Inspector: _________________________ |
| --- | --- |
| Storage Area / Location: _________________________ | Building / Department: _________________________ |
| Inspection Type: ☐ Receiving ☐ Monthly ☐ Quarterly | Total Cylinders Inspected: ___________ |

Part 1: Cylinder Condition

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | Cylinder body — no dents, bulges, gouges, cuts, or excessive corrosion |  |  |
| 2 | No arc strikes, torch burns, or fire/heat damage evidence |  |  |
| 3 | Hydrostatic test date current (stamped on shoulder) |  |  |
| 4 | Valve not bent, broken, or leaking; handwheel operable |  |  |
| 5 | Valve protection cap in place and threaded securely (when not in use) |  |  |
| 6 | Safety relief device present, intact, not tampered with |  |  |
| 7 | Foot ring intact (if equipped); cylinder stands upright |  |  |

Part 2: Labeling and Identification

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | Contents clearly labeled with name of gas |  |  |
| 2 | GHS/HazCom label with hazard pictograms present |  |  |
| 3 | Status marked — Full, In Use, or Empty (MT) |  |  |
| 4 | Empty cylinders clearly identified and segregated from full |  |  |
| 5 | Owner identification markings visible |  |  |

Part 3: Storage Area Conditions

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | All cylinders stored upright and secured (chains, straps, racks) |  |  |
| 2 | Oxygen separated from fuel gases by 20 ft or 5-ft fire barrier |  |  |
| 3 | Storage area well-ventilated and protected from weather |  |  |
| 4 | Temperature maintained below 125°F; away from heat sources |  |  |
| 5 | Storage does not block exits, stairways, or egress paths |  |  |
| 6 | "No Smoking" signs posted where flammable gases stored |  |  |
| 7 | Fire extinguisher accessible in storage area |  |  |

Deficiencies Found

| # | Location / ID | Deficiency Description | Corrective Action | Due Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

WARNING: Never attempt to repair cylinders, valves, or safety relief devices. Tag damaged or leaking cylinders as unserviceable, move to a well-ventilated outdoor area, and notify the gas supplier immediately.

Inspector Signature                                                    Date: _______________

Safety Manager Signature                                                    Date: _______________