Crane Inspection Checklist

Shift / Monthly / Annual Inspection per OSHA 1926.1412-1417 and ASME B30

Date: _______________

Inspector/Operator: _______________

Crane Type: Mobile / Tower / Overhead / Crawler

Make/Model: _______________

Unit ID / Serial #: _______________

Capacity (tons): _______________

Inspection Type: Shift / Monthly / Quarterly / Annual / Comprehensive

Part 1: Structural & Mechanical

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Boom and jib | No cracks, bends, or corrosion. Lattice boom lacing intact. Hydraulic boom cylinders no leaks. Boom pins secured. |  |  |  |
| Wire rope | No kinks, bird-caging, core protrusion, or crushing. End connections secure. 3+ wraps on drum at max extension. No broken wires exceeding criteria. |  |  |  |
| Hooks and blocks | Safety latch closes. No cracks, bending, or throat opening beyond 15%. Hook swivels freely. Block sheaves rotate. |  |  |  |
| Outriggers/crawlers | Outriggers fully extended and set on pads. Crawlers tensioned correctly. No structural damage. Level indicators functional. |  |  |  |
| Turntable and swing | Swing gear lubricated. No excessive play. Swing brake holds. 360-degree rotation smooth. Turntable bolts torqued. |  |  |  |

Part 2: Controls & Safety Devices

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Operator controls | All levers/pedals return to neutral. Functions respond proportionally. No binding or sticking. Labels legible. |  |  |  |
| Brakes | Swing brake holds rated load. Hoist brake holds 125% rated load. Travel brakes stop within spec. Parking brake engages. |  |  |  |
| Load moment indicator (LMI) | Calibrated and functional. Displays radius, load, and capacity. Alarms activate at rated capacity. Anti-two-block functional. |  |  |  |
| Boom angle/length indicators | Boom angle indicator accurate. Telescoping length display correct. Maximum radius alarm functional. |  |  |  |
| Emergency systems | Emergency stop shuts all functions. Backup alarms audible. Fire extinguisher accessible. Horn operational. |  |  |  |

Part 3: Rigging & Site Conditions

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Slings and rigging | Rated for load. No damage, cuts, or kinking. Tags legible. Shackles properly sized and pinned. Softeners used at edges. |  |  |  |
| Ground conditions | Ground compacted and level. Cribbing/mats under outriggers. No underground utilities or voids. Slopes evaluated. |  |  |  |
| Power line clearance | Minimum clearance maintained (20 ft for up to 350kV). Spotter assigned if within table A distance. Utility notified. |  |  |  |
| Lift plan | Written plan for critical lifts. Load weight verified. Rigging plan reviewed. Swing path clear. Communication established. |  |  |  |
| Personnel | Operator certified per 1926.1427. Signal person qualified. Rigger qualified. Lift director designated for critical lifts. |  |  |  |

Crane contact with power lines is one of the leading causes of crane fatalities. Always assume power lines are energized. Maintain minimum clearances per OSHA 1926.1408 Table A. Never operate a crane with defective brakes, non-functional LMI, or damaged wire rope.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Inspector/Operator: ____________________________    Date: ______________    Signature: ____________________________

Crane Manager / Competent Person: ____________________________    Date: ______________    Signature: ____________________________