Excavation & Trenching Inspection Checklist

Daily Competent Person Inspection per OSHA 1926 Subpart P

Date: _______________

Competent Person: _______________

Project / Location: _______________

Excavation Depth: _______________

Soil Classification: Stable Rock / Type A / Type B / Type C

Protective System: Sloping / Benching / Shoring / Shielding / N/A (<5 ft)

Part 1: Pre-Entry & Soil Classification

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Competent person on site | Designated person present with authority to stop work. Trained in soil classification and protective system selection. |  |  |  |
| Soil classification | At least one visual and one manual test performed per Appendix A. Classification documented. Reassessed after rain. |  |  |  |
| Utility location | 811 notification made 48-72 hrs prior. Utilities marked. Hand digging within tolerance zone. |  |  |  |
| Surface conditions | Encumbrances removed/supported. Water diverted. Spoil pile 2+ ft from edge. Adjacent structures evaluated. |  |  |  |
| Excavation permit | Permit issued (if required). Documents location, depth, soil, protective system, competent person. |  |  |  |

Part 2: Protective Systems & Access/Egress

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Protective system installed | System in place for 5+ ft depth per 1926.652. Matches soil type. Extends from bottom to top of excavation. |  |  |  |
| Sloping/benching | Slope angle correct: A=3/4:1, B=1:1, C=1.5:1. No undercutting. No surcharge loads in failure zone. |  |  |  |
| Shoring | Installed per specs. Hydraulics pressurized/locked. No gaps to soil. Timber components sound. |  |  |  |
| Shielding (trench boxes) | Rated for depth/soil. Extends 18 in. above grade. No more than 2 ft excavation below bottom. Backfilled properly. |  |  |  |
| Access and egress | Ladder/stairway/ramp within 25 ft lateral travel for 4+ ft depth. Ladders extend 3 ft above edge. Access clear. |  |  |  |

Part 3: Hazards & Ongoing Monitoring

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Atmospheric hazards | Testing where hazard expected. O2: 19.5-23.5%. Flammable <10% LEL. Toxic below PELs. Ventilation if needed. |  |  |  |
| Water accumulation | No uncontrolled water in excavation. Dewatering equipment monitored. Safety harness if drowning hazard. |  |  |  |
| Adjacent structures | Stability evaluated. Shoring/bracing installed where needed. Monitoring for settlement or cracking. |  |  |  |
| Vehicular traffic | Workers in hi-vis vests. Barriers protecting excavation. Stop logs at edges. Signage directing traffic away. |  |  |  |
| Daily inspection | Inspected before work and after rain/changing conditions. Evidence of cave-in potential documented. Workers removed until corrected. |  |  |  |

NEVER enter an unprotected excavation 5 feet or deeper. A cubic yard of soil weighs approximately 2,700 lbs. Cave-ins can occur without warning. OSHA willful violation penalties can reach $161,323 per violation.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Competent Person: ____________________________    Date: ______________    Signature: ____________________________

Site Superintendent / Safety Manager: ____________________________    Date: ______________    Signature: ____________________________