Fall Protection Equipment Inspection Checklist

| Date: |  | Inspector: |  |
| --- | --- | --- | --- |
| Location/Site: |  | Project #: |  |
| Equipment Type: |  | Mfr/Model: |  |
| Serial #/ID: |  | Date of Mfr: |  |
| Inspection Type: | □ Pre-Use □ Annual/Formal □ Post-Incident □ Return to Service |  |  |

Part 1: Full Body Harness Inspection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Webbing (shoulder, chest, leg, sub-pelvic straps) — No cuts, tears, fraying, abrasion, burns, chemical damage, or UV degradation |  |  |
| 2 | Stitching — No pulled, cut, broken, or missing stitches at all load-bearing connection points; contrast thread visible |  |  |
| 3 | D-Rings (dorsal, sternal, side) — No cracks, distortion, sharp edges, or excessive corrosion; pivots freely; no wear >5% of cross-section |  |  |
| 4 | Buckles and Adjusters — All buckles function properly; tongues seat securely; adjusters hold position under load; no distortion or corrosion |  |  |
| 5 | Grommets and Rivets — All present and intact; not pulled through webbing; no deformation or corrosion |  |  |
| 6 | Labels and Markings — Manufacturer label legible with model, date of manufacture, capacity, and ANSI Z359.11 compliance |  |  |
| 7 | Fall/Impact Indicators — Not deployed; if any indicator shows evidence of impact loading, remove from service immediately |  |  |

Part 2: Lanyard / Energy Absorber Inspection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Lanyard Webbing or Rope — No cuts, burns, abrasion, or chemical damage; rope core not visible through sheath; check full length |  |  |
| 2 | Snap Hooks — Locking type; gate closes completely and lock engages; keeper holds gate closed; no distortion, cracks, or corrosion |  |  |
| 3 | Carabiners — Gate closes and auto-locks properly; no deformation, cracks, or excessive wear at load-bearing surfaces |  |  |
| 4 | Energy Absorber (Shock Pack) — Outer cover intact, no tears or deployment evidence; inner pack not separated or exposed; if deployed, destroy lanyard |  |  |
| 5 | Thimbles and Terminations — Rope thimbles seated properly; splice integrity maintained; stitching at terminations intact |  |  |
| 6 | Labels — Manufacturer, model, capacity, and date of manufacture legible; ANSI Z359.13 compliance marking visible |  |  |

Part 3: Self-Retracting Lifeline (SRL) Inspection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Housing — No cracks, dents, or deformation; all screws, rivets, and bolts tight; swivel attachment rotates freely |  |  |
| 2 | Lifeline (Cable or Webbing) — Cable: no kinks, broken strands, bird-caging, or corrosion; Webbing: no cuts, abrasion, or chemical damage |  |  |
| 3 | Retraction/Extension — Lifeline extends and retracts smoothly through full working length; no binding or hesitation |  |  |
| 4 | Braking Mechanism — Sharp pull engages brake and locks immediately; no slipping or continued extension after engagement |  |  |
| 5 | Connector (Snap Hook) — Locking snap hook gate closes and locks fully; no distortion, corrosion, or damage; spring provides positive closure |  |  |
| 6 | Fall Indicator — Not deployed; if indicator shows impact loading, remove from service and return to manufacturer/authorized service center |  |  |
| 7 | Labels and Service Date — Manufacturer label legible; ANSI Z359.14 compliance marking; if annual service required, verify current date |  |  |

Part 4: Anchorage / Anchor Connector Inspection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Structural Integrity — Anchor point shows no cracks, deformation, or loosening from supporting structure; bolts/welds intact |  |  |
| 2 | Capacity Rating — Rated for at least 5,000 lbs per employee (or part of system with safety factor ≥2); rating label visible |  |  |
| 3 | Anchor Connector — Beam clamps, roof anchors, or connectors properly installed per manufacturer instructions; no slippage or missing parts |  |  |
| 4 | Independence — Anchorage is independent of any anchor used to support/suspend platforms, scaffolding, or material hoists |  |  |
| 5 | Location/Positioning — Positioned to limit free fall to 6 ft or less; prevents swing falls into obstructions; overhead anchorage preferred |  |  |

Deficiencies Found

| # | Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

WARNING: Any fall protection component that has arrested a fall, shows visible damage, or fails inspection must be immediately removed from service and tagged "Do Not Use." Shock-absorbing lanyards with deployed packs must be destroyed. SRLs must be returned to the manufacturer or authorized service center for post-fall inspection. Equipment shall not be used for employee protection until inspected and approved by a competent person per 29 CFR 1926.502(d)(20).

Overall Equipment Status:   □ Approved for Use    □ Removed from Service    □ Requires Manufacturer Service

Inspector: ________________________________________    Date: ______________

Competent Person (annual): ________________________________________    Date: ______________

Safety Manager: ________________________________________    Date: ______________

Provided by Ecesis EHS Software – www.ecesis.net