# Forklift Daily Pre-Shift Inspection Checklist

OSHA 29 CFR 1910.178 | Powered Industrial Trucks

| Date: ________________ | Shift: _______________ | Truck #: _____________ | Hour Meter: __________ |
| --- | --- | --- | --- |
| Operator: ____________ | Dept: ________________ | Type: IC / Electric | Fuel: Gas / LPG / Diesel |

## Part 1: Visual Checks (Engine Off)

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Fluid Leaks | No puddles or wet spots. No hydraulic, oil, coolant, or fuel leaks. |  |  |
| Tires & Wheels | Adequate tread, no cuts/damage, properly inflated. Lug nuts tight. |  |  |
| Forks | No cracks, bends, or excessive wear. Tips aligned. Lock pins in place. |  |  |
| Mast & Chains | Chains lubricated, no kinks or broken links. Mast rails undamaged. |  |  |
| Hydraulic Hoses | No leaks, cracks, abrasion, or bulging. Connections secure. |  |  |
| Overhead Guard | Securely attached, no cracks. All mounting bolts tight. |  |  |
| Load Backrest | Securely attached and undamaged. |  |  |
| Seat & Seatbelt | Seat secure, adjustment works. Seatbelt retracts and latches properly. |  |  |
| Safety Labels | Capacity data plate legible. All warning labels present. |  |  |
| Fluid Levels (IC) | Engine oil, coolant, hydraulic fluid, brake fluid at proper levels. |  |  |
| Fuel System (IC) | Fuel cap secure, no leaks. LPG tank secured, fittings undamaged. |  |  |
| Battery (Electric) | Connector secure, electrolyte adequate, no corrosion. Restraint in place. |  |  |

## Part 2: Operational Checks (Engine Running)

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Service Brake | Adequate stopping power, no pulling. Firm pedal feel. |  |  |
| Parking Brake | Holds on level surface and grades. Engages/releases smoothly. |  |  |
| Steering | Responsive, no excessive play or binding. Full turns smooth. |  |  |
| Horn | Audible and functioning. |  |  |
| Lights | Headlights, taillights, and warning lights all operational. |  |  |
| Backup Alarm | Audible alarm sounds in reverse (if equipped). |  |  |
| Gauges | All within normal range. Warning lights clear after startup. |  |  |
| Mast Operation | Lift, lower, tilt smooth through full range. No jerking or drifting. |  |  |
| Side Shift | Operates smoothly both directions (if equipped). |  |  |
| Unusual Noises | No grinding, clicking, squealing, or excessive vibrations. |  |  |
| Exhaust (IC) | No excessive smoke. Exhaust system intact, no leaks. |  |  |
| Drive/Transmission | Smooth forward/reverse engagement. No slipping or grinding. |  |  |

## Deficiencies & Corrective Actions

| Deficiency Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- |

Forklift Condition:  SAFE to operate  /  UNSAFE – removed from service

Operator Signature: ________________________________    Date: ______________

Supervisor Signature: _______________________________    Date: ______________

If any item fails inspection, immediately remove the forklift from service, tag as Out of Service, and report to your supervisor. Only authorized maintenance personnel may perform repairs.

Provided by Ecesis EHS Software – www.ecesis.net