Hazardous Waste Storage Area Inspection Checklist

| Date: |  |
| --- | --- |
| Inspector: |  |
| Storage Area/Location: |  |
| Generator Status: | LQG / SQG / VSQG |
| Inspection Type: | Weekly / Monthly |
| Total Containers: |  |

Part 1: Container Condition & Management

| Inspection Item | OK / Fail / N/A |
| --- | --- |
| All containers in good condition: no leaks, rust, dents, bulging, or structural damage per 265.171 | ☐ OK ☐ Fail ☐ N/A |
| All containers kept closed during storage per 265.173 (lids, bungs, caps secured) | ☐ OK ☐ Fail ☐ N/A |
| Container material compatible with waste; incompatible wastes separated per 265.172/177 | ☐ OK ☐ Fail ☐ N/A |
| Adequate aisle space for inspection, sampling, and emergency equipment access (min. 30 inches) | ☐ OK ☐ Fail ☐ N/A |
| Ignitable/reactive waste containers at least 50 feet from property line per 265.176; No Smoking signs posted | ☐ OK ☐ Fail ☐ N/A |

Part 2: Labeling, Marking & Dating

| Inspection Item | OK / Fail / N/A |
| --- | --- |
| Each container marked "Hazardous Waste" per 262.17(a)(1)(iv) / 262.16(b)(6); legible and visible | ☐ OK ☐ Fail ☐ N/A |
| Accumulation start date clearly marked on each container; within applicable time limits | ☐ OK ☐ Fail ☐ N/A |
| Hazard indication labels present (DOT, GHS, or EPA characteristic labels) | ☐ OK ☐ Fail ☐ N/A |
| Waste stream contents identifiable; waste codes documented; characterization records on file | ☐ OK ☐ Fail ☐ N/A |
| Satellite containers marked "Hazardous Waste" with contents; volume not exceeding 55 gallons | ☐ OK ☐ Fail ☐ N/A |

Part 3: Storage Area & Containment

| Inspection Item | OK / Fail / N/A |
| --- | --- |
| Secondary containment in place and in good condition (berms, dikes, curbed areas); adequate capacity | ☐ OK ☐ Fail ☐ N/A |
| Containment area free of standing liquids; drain valves closed and locked; rainwater managed | ☐ OK ☐ Fail ☐ N/A |
| Floor/base impervious and free of cracks; no staining; adequate ventilation and lighting | ☐ OK ☐ Fail ☐ N/A |
| Fire extinguisher accessible and inspected; spill cleanup materials (absorbents, PPE) available | ☐ OK ☐ Fail ☐ N/A |
| Storage area secured; "Hazardous Waste" signage posted; access limited to authorized personnel | ☐ OK ☐ Fail ☐ N/A |
| Emergency coordinator designated; contingency plan current; communication equipment accessible | ☐ OK ☐ Fail ☐ N/A |

⚠ WARNING: Exceeding the 90-day (LQG) or 270/180-day (SQG) accumulation time limit subjects the generator to full RCRA storage permit requirements and significant penalties. Transfer or ship waste before time limits expire.

Deficiency Log

| # | Deficiency Description | Corrective Action | Target Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |

| Inspector Signature | Date |
| --- | --- |

| Environmental Manager Signature | Date |
| --- | --- |