Hearing Conservation Inspection Checklist

Per OSHA 29 CFR 1910.95 and NIOSH Best Practices

Date: _______________

Inspector: _______________

Facility / Area: _______________

Department: _______________

Noise Exposure Level (TWA): _______________

Inspection Type: Monthly / Quarterly / Annual

Part 1: Noise Monitoring & Engineering Controls

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Noise monitoring | Area/personal monitoring current (within 2 years or after process change). TWA documented. Action level (85 dBA) areas identified. |  |  |  |
| Noise map / signage | Noise hazard map current. High-noise areas posted with signage. HPD requirement signs at entry points. |  |  |  |
| Engineering controls | Enclosures, barriers, damping treatments intact. Equipment mounting isolators in good condition. No new noise sources. |  |  |  |
| Equipment maintenance | Bearings, belts, gears lubricated per schedule. Worn parts replaced. Mufflers/silencers intact. Vibration levels normal. |  |  |  |
| Administrative controls | Worker rotation schedules followed. Quiet break areas available. Job duration limits enforced in high-noise areas. |  |  |  |

Part 2: Hearing Protection Devices

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| HPD availability | Variety of HPDs available (plugs, muffs, banded). Dispensers stocked in high-noise areas. Multiple sizes available. |  |  |  |
| HPD condition | Earmuff cushions not hardened/cracked. Headband tension adequate. Foam plugs not reused. Custom molds intact. |  |  |  |
| NRR adequacy | HPD NRR sufficient for exposure level. De-rated NRR still provides adequate protection. Dual protection where TWA > 100 dBA. |  |  |  |
| Proper fit/use | Workers observed wearing HPDs correctly. Plugs fully inserted. Muffs sealed around ears. No gaps from glasses/hair. |  |  |  |
| HPD storage | Clean storage cases/bags provided. Reusable HPDs cleaned regularly. Replacement schedule followed. No contamination. |  |  |  |

Part 3: Training, Audiometry & Records

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Annual training | Training completed for all workers in HCP. Topics: noise effects, HPD use/care, audiometric testing purpose. Attendance documented. |  |  |  |
| Baseline audiograms | Baseline within 6 months of first exposure (1 year with HPD). 14-hour quiet period observed. Results on file. |  |  |  |
| Annual audiograms | Completed within 12 months of prior test. Audiometer calibrated. Testing by qualified technician. |  |  |  |
| Standard threshold shift | STS cases identified and reviewed. Workers notified within 21 days. HPD refitted or upgraded. Follow-up audiogram scheduled. |  |  |  |
| Recordkeeping | Audiometric records retained for duration of employment. Noise exposure records retained 2 years. Training records current. |  |  |  |

OSHA requires a hearing conservation program when workers are exposed to noise at or above 85 dBA TWA (8-hour). NIOSH recommends action at 85 dBA using a 3 dB exchange rate. Noise-induced hearing loss is permanent and irreversible. Standard Threshold Shifts must be evaluated and recorded on OSHA 300 log if work-related.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Inspector: ____________________________    Date: ______________    Signature: ____________________________

Safety Manager / Industrial Hygienist: ____________________________    Date: ______________    Signature: ____________________________