Hot Work Permit Inspection Checklist

Per OSHA 1910.252 and NFPA 51B

Date: _______________

Permit Authorizing Individual (PAI): _______________

Hot Work Operator: _______________

Fire Watch Person: _______________

Location / Area: _______________

Type of Hot Work: Welding / Cutting / Brazing / Grinding / Other

Start Time: _______________

Permit Expiration: _______________

Part 1: Pre-Work Area Inspection

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| 35-ft combustible clearance | All movable combustibles removed to 35 ft. Immovable covered with fire-resistant material. Floor swept clean. |  |  |  |
| Wall/floor/ceiling openings | Openings within 35 ft sealed. Cracks/gaps in fire barriers covered. Opposite side of walls inspected. |  |  |  |
| Flammable atmosphere | No flammable vapors/gases/dust. Monitoring performed. LEL below 10%. Containers of flammables closed. |  |  |  |
| Fire suppression systems | Sprinklers in service. Heads unobstructed. If impaired, hot work prohibited until restored. |  |  |  |
| Smoke detection | Detectors identified and temporarily covered if needed. Plan to restore after work. Building mgmt notified. |  |  |  |

Part 2: Permit, Equipment & PPE

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Hot work permit issued | Written permit with date, time, location, operator, fire watch, precautions, expiration. Posted at location. |  |  |  |
| Fire extinguisher | Min 2A:10BC within immediate reach. Fully charged. Operator and fire watch trained in use. |  |  |  |
| Welding/cutting equipment | Safe operating condition. Cables/hoses undamaged. Ground clamp secure. Regulators/arrestors working. |  |  |  |
| PPE | Welding helmet correct shade. FR clothing. Leather gloves. Safety-toed boots. Hearing/respiratory as needed. |  |  |  |
| Welding screens/barriers | Screens protecting nearby workers from arc/sparks/UV. Screens rated for welding. Warning signs posted. |  |  |  |

Part 3: Fire Watch & Post-Work

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Fire watch assigned | Dedicated person, no other duties. Trained in extinguisher use. Authority to stop work. Full area visibility. |  |  |  |
| Fire watch equipment | Extinguisher within reach. Communication device. Flashlight for concealed areas. Log/checklist for records. |  |  |  |
| Post-work monitoring | Fire watch maintained 30 min (OSHA) / 60 min (NFPA 51B) minimum. Extended in high-risk areas. No hot spots. |  |  |  |
| Post-work documentation | Completion time recorded. Fire watch signs off. Permit closed and filed. Detectors restored to service. |  |  |  |
| Multi-level monitoring | Areas above/below checked for spark travel. Pipe chases/wall penetrations inspected. No debris on combustibles. |  |  |  |

NFPA reports an average of 4,580 structure fires per year from hot work. Sparks can travel 35+ feet horizontally and much further through openings. Never leave a hot work area without completing the fire watch period. Minimum: 30 min OSHA, 60 min NFPA 51B.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Permit Authorizing Individual (PAI): ____________________________    Date: ______________    Signature: ____________________________

Hot Work Operator: ____________________________    Date: ______________    Signature: ____________________________

Fire Watch Person: ____________________________    Date: ______________    Signature: ____________________________