# Lockout/Tagout (LOTO) Periodic Inspection Checklist

OSHA 29 CFR 1910.147(c)(6) | Annual Periodic Inspection

| Date: ________________ | Inspector: ___________ | Location: ____________ |
| --- | --- | --- |
| Machine/Equipment: ____________________ | Procedure #: _________ | Authorized Employee Observed: _________ |

## Part 1: Written Procedure Review

| Inspection Item | What to Verify | Yes | No |
| --- | --- | --- | --- |
| Procedure identification | Procedure identifies specific machine/equipment, location, and all types of energy to be controlled. |  |  |
| Energy source identification | All energy sources (electrical, mechanical, hydraulic, pneumatic, chemical, thermal, gravitational) are listed. |  |  |
| Isolation points | All energy-isolating devices are correctly identified. No new energy sources have been added since last review. |  |  |
| Shutdown sequence | Written shutdown steps are accurate and match current equipment configuration. |  |  |
| LOTO device placement | Procedure specifies where locks/tags are applied for each isolation point. |  |  |
| Stored energy dissipation | All forms of stored/residual energy are addressed with specific methods for dissipation or restraint. |  |  |
| Verification of isolation | Procedure requires verification that energy is isolated (try controls, test with instruments, check gauges). |  |  |
| Release from LOTO | Steps for removing locks/tags, clearing work area, notifying affected employees, and restoring equipment. |  |  |
| Group LOTO provisions | If applicable, group lockout procedure addresses personal lock requirements and coordination. |  |  |
| Shift change procedures | If applicable, procedures for transferring LOTO protection between shifts are documented. |  |  |

## Part 2: Employee Knowledge and Execution

| Inspection Item | What to Verify | Yes | No |
| --- | --- | --- | --- |
| Authorized employee knowledge | Employee can explain the procedure and the purpose of each step. |  |  |
| Correct procedure execution | Inspector observed employee performing LOTO. Each step matched written procedure. |  |  |
| Proper device application | Employee applied correct lock/tag to each isolation point identified in the procedure. |  |  |
| Energy verification | Employee verified energy isolation before beginning work (tried controls, tested circuits, etc.). |  |  |
| Affected employee awareness | Affected employees understand LOTO purpose and know not to restart locked/tagged equipment. |  |  |
| Release procedure followed | Employee followed proper steps for removing LOTO devices and restoring equipment. |  |  |

## Part 3: Devices and Equipment

| Inspection Item | What to Verify | Yes | No |
| --- | --- | --- | --- |
| Lockout devices | Locks are durable, standardized, substantial, and labeled with authorized employee identity. |  |  |
| Tagout devices | Tags are durable, standardized, legible, include employee identity, and include warning legend. |  |  |
| Device exclusivity | LOTO devices used exclusively for energy control (not used for tool storage, lockers, etc.). |  |  |
| Energy-isolating devices | All isolation devices (breakers, disconnects, valves) operational and capable of being locked out. |  |  |
| Labeling | Energy isolation points on equipment clearly labeled, legible, and matching the written procedure. |  |  |
| Device inventory | Adequate supply of locks, tags, hasps, and other devices available. |  |  |

## Deficiencies & Corrective Actions

| Deficiency Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- |

Overall Finding:  Procedure adequate and being followed  /  Deficiencies identified (see above)

Retraining Required:  Yes  /  No       If yes, list employees: ___________________________________

Inspector Signature: ________________________________    Date: ______________

Safety Manager: ____________________________________    Date: ______________

If deviations or inadequacies are found, correct the procedure and retrain all affected employees before the procedure is used again. Document retraining with employee name and date per 1910.147(c)(7).

Provided by Ecesis EHS Software – www.ecesis.net