# Ladder Inspection Checklist

OSHA 29 CFR 1910.23 (General Industry) | 29 CFR 1926.1053 (Construction)

| Date: ________________ | Inspector: ___________ | Location: ____________ | Ladder ID: ___________ |
| --- | --- | --- | --- |
| Ladder Type: Step / Extension / Fixed / Job-Made | Material: Aluminum / Fiberglass / Wood | Duty Rating: _________ |

## Part 1: Structural Inspection

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Side rails | No cracks, splits, bends, or corrosion. Fiberglass not chalky or delaminated. Wood not painted (opaque coatings hide defects). |  |  |
| Rungs/steps | All present, secure, and evenly spaced. No bending, cracking, or excessive wear. Slip-resistant surface intact. |  |  |
| Rung-to-rail connections | Tight and secure. No wobbling, looseness, or visible damage at connection points. |  |  |
| Feet/shoes | Non-slip feet/pads present and in good condition. No excessive wear, missing, or damaged foot pads. |  |  |
| Spreader/locking device | Stepladder spreaders lock fully open. No bent, broken, or missing locking mechanisms. |  |  |
| Extension locks (dawgs) | Engage securely at each rung. Spring-loaded locks snap into place. No bent or worn teeth. |  |  |
| Rope and pulley | Extension ladder rope intact, not frayed. Pulley rotates freely. Rope securely attached. |  |  |
| Safety labels | Capacity/duty rating label legible. Warning labels present and readable. ANSI compliance markings visible. |  |  |
| Hardware | All bolts, rivets, screws, and fasteners tight and present. No missing or stripped hardware. |  |  |
| Overall condition | No unauthorized modifications or repairs (tape, wire, screws). Ladder clean, free of oil, grease, and wet paint. |  |  |

## Part 2: Setup and Use Verification

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Stable surface | Ladder on firm, level surface. Not on boxes, barrels, or other unstable bases. |  |  |
| 4-to-1 rule (extension) | Base set out 1 foot for every 4 feet of working length (75.5-degree angle). |  |  |
| 3-foot extension | Extension ladder extends at least 3 feet above upper landing surface. |  |  |
| Secured/tied off | Ladder secured at top, bottom, or both to prevent displacement. Stabilizers used if needed. |  |  |
| Electrical clearance | Nonconductive ladder used near electrical equipment. Minimum clearances from power lines maintained. |  |  |
| Clear access | Area at top and bottom of ladder clear. No obstructions, tripping hazards, or traffic. |  |  |
| Proper use | Ladder used only for designed purpose. Not used as scaffolding, bridge, or in closed/folded position. |  |  |

## Deficiencies & Corrective Actions

| Deficiency Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- |

Ladder Condition:  SAFE to use  /  UNSAFE – removed from service

Inspector Signature: ________________________________    Date: ______________

Supervisor Signature: _______________________________    Date: ______________

If any structural defect is found, immediately tag the ladder "Do Not Use" and remove from service. Do not attempt field repairs. Defective ladders must be repaired by the manufacturer or a qualified person, or destroyed.

Provided by Ecesis EHS Software – www.ecesis.net