MACHINE GUARDING INSPECTION CHECKLIST

Per OSHA 29 CFR 1910.212 and 1910.219

| Date: _______________ | Inspector: _________________________ |
| --- | --- |
| Machine ID: _________________________ | Machine Type: _________________________ |
| Location / Department: _________________________ | Make / Model: _________________________ |
| Inspection Type: ☐ Pre-Use ☐ Monthly ☐ Post-Maintenance | Serial #: _________________________ |

Part 1: Barrier and Fixed Guards

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | Guard integrity — no cracks, breaks, holes, or deformation |  |  |
| 2 | Secure attachment — all fasteners in place and tight, no shifting |  |  |
| 3 | Openings and gaps — no access to danger zone (OSHA Table O-10) |  |  |
| 4 | Guard does not create new hazards (sharp edges, pinch points) |  |  |
| 5 | Guard allows safe operation, feeding, and material removal |  |  |

Part 2: Point of Operation Guarding

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | Guard fully covers point of operation during operating cycle |  |  |
| 2 | Adjustable guards properly set for current stock size |  |  |
| 3 | Self-adjusting guards open/close correctly, spring tension adequate |  |  |
| 4 | Interlocked guards — machine stops when guard opened (test) |  |  |
| 5 | Two-hand controls require concurrent activation, cannot be defeated |  |  |

Part 3: Safety Devices and Presence Sensing

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | Light curtains — all beams functional, proper alignment (test) |  |  |
| 2 | Safety mats / edges — activate on contact, no dead spots (test) |  |  |
| 3 | Emergency stop buttons — accessible, functional, latching (test) |  |  |
| 4 | Safety relays / controllers — normal status, no fault codes |  |  |
| 5 | Reset requires deliberate action after E-stop activation |  |  |

Part 4: Power Transmission Guards (1910.219)

| # | Inspection Item | OK / Fail | Notes |
| --- | --- | --- | --- |
| 1 | Belts and pulleys within 7 ft of floor — fully guarded |  |  |
| 2 | Chains and sprockets — enclosed or guarded, no exposed teeth |  |  |
| 3 | Rotating shafts within 7 ft — guarded, no exposed keyways or setscrews |  |  |
| 4 | Gears — enclosed or guarded to prevent finger contact |  |  |
| 5 | Flywheels — guarded if any part within 7 ft of floor |  |  |

Deficiencies Found

| # | Location / ID | Deficiency Description | Corrective Action | Due Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

WARNING: If any guard is missing, bypassed, or not functioning, tag the machine out of service immediately. Guards must be reinstalled and verified before returning to production. Follow lockout/tagout per OSHA 1910.147 for all maintenance.

Inspector Signature                                                    Date: _______________

Supervisor / Safety Manager Signature                                                    Date: _______________