Personal Protective Equipment (PPE) Inspection Checklist

| Date: |  | Inspector: |  |
| --- | --- | --- | --- |
| Location/Area: |  | Department: |  |
| Inspection Type: |  | Hazard Assessment Date: |  |

Part 1: Eye and Face Protection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Lenses — No cracks, chips, scratches, or pitting; correct shade/tint; seated securely in frame |  |  |
| 2 | Frame and Temples — No cracks, bending, or distortion; proper tension and fit; side shields present if required |  |  |
| 3 | Straps/Headband — Elastic, adjustable, no fraying; face shield ratchet functional; secure fit |  |  |
| 4 | Ventilation — Goggle vents not blocked; indirect vents intact for splash protection |  |  |
| 5 | Markings — ANSI Z87.1 marking visible on lens and frame; impact rating (Z87+) present if required |  |  |

Part 2: Head Protection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Shell — No cracks, dents, gouges, or penetration; no chalking/brittleness from UV; no unauthorized modifications |  |  |
| 2 | Suspension System — All attachments connected; headband adjustable; crown straps intact; proper standoff distance |  |  |
| 3 | Brim and Peak — No cracks or chips; no warping or deformation |  |  |
| 4 | Electrical Rating — Class E or G appropriate for exposure; no conductive materials added |  |  |
| 5 | Markings and Age — ANSI Z89.1 marking visible; type and class legible; within manufacturer service life (typically 2–5 years) |  |  |

Part 3: Hand Protection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Physical Condition — No holes, tears, punctures, or thinning; seams intact; no excessive wear |  |  |
| 2 | Chemical Resistance — No swelling, stiffening, softening, or discoloration from chemical exposure |  |  |
| 3 | Cut Resistance — ANSI/ISEA 105 cut level appropriate; no cuts or nicks; coating intact |  |  |
| 4 | Flexibility and Fit — Maintains dexterity; proper size; cuff length adequate |  |  |
| 5 | Insulation (if applicable) — Thermal/electrical insulation intact; electrical gloves tested per schedule |  |  |

Part 4: Foot Protection

| # | Inspection Item | OK | Fail |
| --- | --- | --- | --- |
| 1 | Safety Toe — Toe cap not exposed, dented, or cracked; upper material intact over cap |  |  |
| 2 | Soles — Adequate tread depth; no separation from upper; puncture-resistant plate intact if applicable |  |  |
| 3 | Upper and Closures — No holes, tears, or excessive wear; laces/buckles/zippers functional |  |  |
| 4 | Electrical Hazard Rating — EH-rated: sole and heel intact; no conductive modifications |  |  |
| 5 | Markings — ASTM F2413 compliance marking visible; protection ratings appropriate for hazards |  |  |

Part 5: High-Visibility and Body Protection

| # | Inspection Item | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| 1 | High-Visibility Garment — ANSI/ISEA 107 class appropriate; retroreflective strips intact; fluorescent material not faded |  |  |  |
| 2 | Protective Clothing — Chemical suits/FR clothing: no holes, tears, or contamination; seams sealed; closures functional |  |  |  |
| 3 | Hearing Protection — Earplugs: no hardening or cracking; earmuffs: cushions pliable, headband pressure adequate, NRR rating appropriate |  |  |  |

Deficiencies Found

| # | Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

WARNING: Defective or damaged PPE shall not be used per 29 CFR 1910.132(e). Remove from service and replace immediately. Do not attempt to repair PPE unless authorized by the manufacturer.

Inspector: ________________________________________    Date: ______________

Safety Manager: ________________________________________    Date: ______________

Provided by Ecesis EHS Software – www.ecesis.net