Pressure Vessel Inspection Checklist

Visual and Periodic Inspection per ASME BPVC, API 510, and OSHA 1910.169

Date: _______________

Inspector: _______________

Vessel ID / Tag Number: _______________

Location: _______________

MAWP / Design Pressure: _______ psig

Vessel Contents: _______________

Inspection Type: External / Internal / On-Stream / Annual

Part 1: External Visual Inspection

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Shell and heads | Free from corrosion, pitting, blistering, bulging, cracks. No distortion. Insulation intact. Coating/paint adequate. |  |  |  |
| Nozzles and connections | All connections tight, no leaks. Flanges properly bolted. Welded connections no cracks. Blanked connections secure. |  |  |  |
| Supports and foundations | Supports intact and structurally sound. Foundation no cracking or settling. Anchor bolts tight. No vessel movement. |  |  |  |
| Nameplate and markings | ASME nameplate present and legible. MAWP, manufacturer, serial number visible. National Board number present. |  |  |  |
| Piping and connections | Connected piping properly supported. Expansion joints in good condition. Valves accessible. No vibration-induced fatigue. |  |  |  |

Part 2: Safety Devices & Instrumentation

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Pressure relief valves | Present and properly sized for MAWP. Set pressure verified. Discharge piping clear. No block valves between vessel and PRV. Seals intact. |  |  |  |
| Pressure gauges | Present and readable. Dial range appropriate. Connection not blocked. Reading consistent. Calibration current. |  |  |  |
| Temperature instruments | Gauges/sensors functioning. Thermowells intact. Readings consistent. Alarms functional where installed. |  |  |  |
| Drain valves | At lowest point. Operable, not seized. Line clear. Automatic traps functional. Regular draining schedule maintained. |  |  |  |
| Level indicators | Present and functioning. Glass not cracked. Isolation valves operable. Alarms functional. Readings consistent. |  |  |  |

Part 3: Internal Inspection & Documentation

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Internal surface condition | Free from corrosion, pitting, erosion, cracking. UT thickness at CMLs. Wall above minimum. Corrosion rate calculated. Welds inspected. |  |  |  |
| Internal components | Internals secure and in good condition. No loose parts. Linings intact. Cathodic protection adequate. |  |  |  |
| Manways and access | Covers and gaskets in good condition. Seating surfaces not damaged. Bolting adequate. Confined space procedures followed. |  |  |  |
| Thickness records | UT measurements documented at all CMLs. Compared to previous readings. Minimum thickness calculated. Next interval established. |  |  |  |
| Inspection documentation | ASME data report on file. Previous reports organized. Repair/alteration records. PRV test records current. Jurisdictional certificates current. |  |  |  |

Remove any pressure vessel from service immediately if operating pressure exceeds MAWP, wall thickness is below minimum required, safety relief devices are non-functional or blocked, or visual inspection reveals cracks, bulging, or active leaks. Pressure vessel failures can result in catastrophic explosions and fatalities.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Inspector: ____________________________    Date: ______________    Signature: ____________________________

Facility Manager / Engineer: ____________________________    Date: ______________    Signature: ____________________________