# Scaffold Inspection Checklist

OSHA 29 CFR 1926.451 (Construction) | 29 CFR 1910.28 (General Industry)

| Date: ________________ | Inspector: ___________ | Location: ____________ | Scaffold ID: _________ |
| --- | --- | --- | --- |
| Type: Supported / Suspended / Mobile / Tubular | Max Height: __________ | Max Load: ____________ |

## Part 1: Foundation and Structure

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Base plates/mudsills | Base plates on all legs. Mudsills adequate for soil conditions. Firm, level foundation. |  |  |
| Screw jacks/leveling | Screw jacks used on uneven surfaces. Properly adjusted. Not extended beyond manufacturer limits. |  |  |
| Bracing | Cross bracing, horizontal bracing, and diagonal bracing installed per manufacturer specs. All pins and clips in place. |  |  |
| Frame connections | All frame connections secure. Coupling pins locked. No missing or damaged connectors. |  |  |
| Plumb and level | Scaffold is plumb, square, and level. No visible leaning, racking, or twisting. |  |  |
| Planking/decking | Scaffold-grade planks or manufactured decking. Extends 6-12 inches past supports. No gaps greater than 1 inch. Cleated or restrained. |  |  |
| Ties and anchors | Scaffold tied to building at required intervals (per 1926.451(c)(1)). Anchors secure. No missing tie-ins. |  |  |
| Capacity rating | Load capacity posted or communicated. Not exceeded. Includes weight of workers, tools, and materials. |  |  |

## Part 2: Fall Protection and Access

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Guardrails | Top rail 38-45 inches above platform. Mid rail at mid-height. Toeboard at least 3.5 inches high. All rails secure. |  |  |
| Guardrail strength | Top rail can withstand 200 lbs downward or outward force. Mid rail can withstand 150 lbs downward or outward force. |  |  |
| Access/egress | Proper access provided (ladder, stairway, ramp, or direct access). Climbing cross braces prohibited. |  |  |
| Ladder access | Access ladders extend 3 feet above platform. Secured against displacement. Rungs uniformly spaced. |  |  |
| Openings in platform | All openings properly guarded or covered. No gaps through which tools or materials could fall. |  |  |
| Falling object protection | Toeboards installed. Screens or debris nets installed where workers below. Hard hat area designated. |  |  |

## Part 3: Electrical and Environmental

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Power line clearance | Minimum clearances from power lines maintained per 1926.451(f)(6). Insulated lines: 3 feet. Uninsulated up to 300V: 3 feet. Higher voltage: 10+ feet. |  |  |
| Weather conditions | Scaffold safe to use in current weather. No ice or snow accumulation. High winds assessed (40+ mph requires work stoppage for most scaffolds). |  |  |
| Tag/inspection status | Current inspection tag displayed showing date and competent person name. No expired or missing tags. |  |  |

## Deficiencies & Corrective Actions

| Deficiency Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- |

Scaffold Condition:  SAFE to use  /  UNSAFE – removed from service

Competent Person: ________________________________    Date: ______________

Supervisor Signature: _______________________________    Date: ______________

If any deficiency is found that affects structural integrity or fall protection, immediately prohibit scaffold use, barricade the area, and tag as unsafe. Do not allow use until a competent person verifies all deficiencies have been corrected.

Provided by Ecesis EHS Software – www.ecesis.net