Stormwater BMP Inspection Checklist

Per NPDES Permit and SWPPP Requirements

Date: _______________

Inspector: _______________

Facility / Site Name: _______________

NPDES Permit #: _______________

Weather Conditions: _______________

Inspection Type: Routine / Post-Storm / Quarterly / Annual

Part 1: Erosion & Sediment Controls

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Silt fences | Fabric intact, no tears/gaps/undermining. Posts anchored. Sediment below 1/3 height. No bypass flow around ends. |  |  |  |
| Sediment basins/traps | Below 50% sediment capacity. Outlet functioning. Embankment stable. Emergency spillway clear. No oil sheen. |  |  |  |
| Inlet protection | Storm drain protection in place. No bypass flow. Sediment not blocking >25% capacity. Devices anchored. |  |  |  |
| Stabilized entrances | Aggregate/paving intact. No sediment tracking onto public roads. Entrance draining back into site. |  |  |  |
| Slope stabilization | Seeding/mulching applied per schedule. Erosion blankets anchored. No rills or gullies forming. Stockpiles covered. |  |  |  |

Part 2: Good Housekeeping & Material Management

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Material storage | Chemicals/fuels covered. Secondary containment in place. No leaks/spills. Materials away from storm drains. |  |  |  |
| Waste management | Dumpsters closed/covered. No overflow. Debris contained. Washout areas properly contained. |  |  |  |
| Vehicle/equipment areas | Designated fueling areas with spill controls. Drip pans under leakers. No stains on ground. |  |  |  |
| Outfall monitoring | All outfalls accessible. No unauthorized discharges. No sheen/discoloration/foam/odor. Structures intact. |  |  |  |
| Spill prevention | Spill kits available near storage. Response plan posted. Employees trained. Spill history log current. |  |  |  |

Part 3: Structural BMPs & Post-Construction

| Inspection Item | What to Check | OK | Fail | N/A |
| --- | --- | --- | --- | --- |
| Detention/retention ponds | Embankment stable. Outlet functioning. Trash racks clear. Vegetation maintained. Sediment below design limit. |  |  |  |
| Bioswales/channels | Channel dimensions maintained. Vegetation healthy. Check dams intact. No sediment impeding flow. |  |  |  |
| Infiltration practices | Draining within design drawdown time. No standing water beyond retention period. Pretreatment functioning. |  |  |  |
| Manufactured devices | Separators/filters inspected per schedule. No bypass during design storms. Media replaced at intervals. |  |  |  |
| Conveyance systems | Channels/ditches clear. Pipe inlets/outlets clear. Riprap stable. Culverts flowing freely. |  |  |  |

NPDES permit violations can result in Clean Water Act penalties up to $64,618 per day per violation. All BMP deficiencies must be corrected within timeframes specified in the permit. Document all corrective actions and update the SWPPP.

Deficiencies Found

| Item # | Deficiency Description | Priority | Corrective Action |
| --- | --- | --- | --- |

Inspector: ____________________________    Date: ______________    Signature: ____________________________

Stormwater Program Manager: ____________________________    Date: ______________    Signature: ____________________________